Course: CSE302 Computer Organization and Architecture

Group Project: Design a five-cycle non-pipelined RISC processor using Verilog HDL

- Your task is to design a five-cycle non-pipelined RISC processor using Verilog HDL, for the new instruction set and instruction format given in this document

  - You may use the Logisim based processor design provided in the class only as the reference for basic understanding purposes. (The Instruction set for this Verilog processor implementation, is very different from the Logisim processor, though most fundamentals/thought process remains the same)

- Note, in the Verilog HDL design of the processor, all FFs (DFF based modules or clk based memory) will have Reset

- Precisely follow the names of the Verilog modules and their ports given here.

Let me know, in case you notice any errors or misprints.

Instruction Set

Instruction Format (total 32-bits)

| OPCODE [31:24] | Destination [23:20] | Source-1 [19:16] | Source-2 [15:12] | IMM [11:0] Used as: - ADDR = 12-bit direct address for DataMemory OR 12-bit direct address for Jump instructions - IMM [3:0] used as Source-3 (for reading RS3) - IMM[7:0] used as Immediate-Data (referred to as IMMDATA) - IMM [3:0] used as the address for Input and Output Ports (referred to as ADDR4) |
| --- | --- | --- | --- | --- |

Timing Phases: Typical (IF/ID/EX/MEM/WB)

Acronyms:

- IMMDATA = IMM[7:0]

- ADDR = IMM[11:0]

- XP = X-Pointer (12-bits) = {RS1[3:0], RS2[7:0]}

- For finding RS3, the Source-3 = IMM[3:0]

- ADDR4 = IMM[3:0]

| OPCODE | Mnemonic | Flags affected |
| --- | --- | --- |
| 00H | NOP | None |
| 01H | ADD RD ← RS1 + RS2 | C, Z |
| 02H | ADDI RD ← RS1 + IMMDATA | C, Z |
| 03H | ADDC RD ← RS1 + RS2 + C | C, Z |
| 04H | SUB RD ← RS1 + 1’sComp(RS2) + 1 | C, Z |
| 05H | SUBI RD ← RS1 + 1’sComp(IMMDATA) + 1 | C, Z |
| 06H | SUBC RD ← RS1 + 1’sComp(RS2) + C | C, Z |
| 07H | CMP RS1 + 1’sComp(RS2) + 1 | C, Z |
| 08H | CMPI RS1 + 1’sComp(IMMDATA) + 1 | C, Z |
| 09H | AND RD ← RS1 & RS2 | Z |
| 0AH | ANDI RD ← RS1 & IMMDATA | Z |
| 0BH | OR RD ← RS1 ¦ RS2 | Z |
| 0CH | ORI RD ← RS1 ¦ IMMDATA | Z |
| 0DH | EXOR RD ← RS1 ^ RS2 | Z |
| 0EH | EXORI RD ← RS1 ^ IMMDATA | Z |
| 0FH | INC RD ← RS1 + 01H | Z |
| 10H | DEC RD ← RS1 + 1’sComp(01H) + 1 | Z |
| 11H | MOVI RD ← IMMDATA |  |
| 12H | MOV RD ← RS1 |  |
| 13H | MOVF RD ← {C, 6’b00000, Z} |  |
| 14H | LOAD RD ← M[ADDR] |  |
| 15H | LOADI RD ← M[XP] |  |
| 16H | STORE M[ADDR] ← RS1 |  |
| 17H | STOREI M[XP] ← RS3 |  |
| 18H | LRS RD[7:0] ← {1’b0, RS1[7:1]} (logical shift right by 1-bit) | C = RS1[0], Z |
| 19H | ARS RD[7:0] ← {RS1[7], RS1[7:1]} (arithmetic shift right by 1-bit) | C = RS1[0], Z |
| 1AH | LLS RD[7:0] ← {RS1[6:0],1’b0} (logical shift left by 1-bit) | C = RS1[7], Z |
| 1BH | ROTR RD[7:0] ← {RS1[0], RS1[7:1]} (rotate right by 1-bit) | C = RS1[0], Z |
| 1CH | ROTL RD[7:0] ← {RS1[6:0], RS1[7]} (rotate left by 1-bit) | C = RS1[7], Z |
| 1DH | ROTRC {C, RD[7:0]} ← {RS1[0], C, RS1[7:1]} (rotate right with C by 1-bit) | C = RS1[0], Z |
| 1EH | ROTLC {C, RD[7:0]} ← {RS1[7:0], C} (rotate left with C by 1-bit) | C = RS1[7], Z |
| 1FH | JMPZ ADDR |  |
| 20H | JMPNZ ADDR |  |
| 21H | JMPC ADDR |  |
| 22H | JMPNC ADDR |  |
| 23H | CALL ADDR |  |
| 24H | RET |  |
| 25H | PUSH Stack ← RS1 |  |
| 26H | POP RD ← Stack |  |
| 27H | unsignedMULT {Stack, RD}← RS1 x RS2 (lower-order byte written in WB to RD) (higher-order byte written to Stack in MEM) The programmer may in subsequent instruction POP to a register the high-order byte of the answer |  |
| 28H | IN RD ← PORT[ADDR4] Input ports have address 0H, 1H, 2H, 3H |  |
| 29H | OUT PORT[ADDR4] ← RS1 Output ports have address 8H, 9H, AH, BH |  |
| 2AH | JMP ADDR (unconditional jump) |  |

Notes:

- There are 16 registers, R0 to R15, each register is 8-bits

- ALU is 8-bits; There are two operands, and each operand is 8-bits

- Address bus for Instruction memory is 12-bits

- Address bus for Data memory is 12-bits

- The internal address bus (and mechanism of SP) of Stack is 8-bits

- The Address bus for I/O ports are 4-bits

- At many places, in your design you will need very large MUXs (because your Opcode is 8-bits) ☹, examples:

  - MUX256_to1_1bit for each control signal

  - MUX256_to1_8bit for operands selection MUXs, to the left of the ALU

  - MUX256_to1_8bit for answer selection MUX inside ALU

  - MUX256_to1_12bit for selecting the address for DataMemory (SRAM4096) or for the ProgramCounter

  - For your reference and understanding, the Verilog design of such large MUXs (including large 8to256 decoder) can be found on:

    - https://edaplayground.com/x/9evy

    - https://edaplayground.com/x/727U

    - https://edaplayground.com/x/abub

- Create processor.v file with the following module

| Module Name | Inputs | Outputs | Comments |
| --- | --- | --- | --- |
| RISCprocessor | clk Reset [7:0] InpExtWorld1 [7:0] InpExtWorld2 [7:0] InpExtWorld3 [7:0] InpExtWorld4 | [7:0] OutExtWorld1 [7:0] OutExtWorld2 [7:0] OutExtWorld3 [7:0] OutExtWorld4 | (Overall Structural coding) There will be several modules instantiated within this module; Some important instances are mentioned below: RegisterFile InstMEM4096 SRAM4096 Stack256 ALU8 TimingGen ProgCounter INport OUTport ControlLogic & some MUX32to1_8bit |

- Create subcomponents.v file with the following modules [Any smaller modules not mention in the table below, will have to be created by you – that is implicit]

| Module Name | Inputs | Outputs | Comments/Guidance |
| --- | --- | --- | --- |
| eightbitRegwithLoad | Clk Reset load [7:0] Datain | [7:0] Dataout | Code already available with you in Lab5 Remember that, this module can also be converted to (or used as) an 8-bit FF by simply passing 1’b1 to load when instantiating it in another module |
| RegisterFile | Clk Reset RegFileRead RegFileWrite [7:0] Datain [3:0] Source1 [3:0] Source2 [3:0] Source3 [3:0] Destin | [7:0] Dataout1 [7:0] Dataout2 [7:0] Dataout3 | Structural coding preferred, with some important instances: 16 instances of eightbitRegwithLoad 1 instance of Decoder4to16_withE 3 instances of MUX16to1_8bit 3 instances of eightbitRegwithLoad Dataout3 used for STOREI instruction |
| InstMEM4096 | Clk Reset [11:0] Address InstRead | [31:0] Dataout [7:0] Opcode [3:0] Destin [3:0] Source1 [3:0] Source2 [3:0] Source3 [11:0] ADDR [7:0] ImmData [3:0] ADDR4 | (Behavioral coding) The memory variable would be declared as: reg [31:0] instmemory [0:4095] Use initial block and $readmemb to load your assembly language program (instructions in binary format) |
| SRAM256 | Clk Reset [7:0] Address SRAMRead SRAMWrite [7:0] Datain | [7:0] Dataout | (Behavioral coding) The memory variable would be declared as: reg [7:0] datamemsmall [0:255] |
| SRAM4096 | Clk Reset [11:0] Address SRAMRead SRAMWrite [7:0] Datain | [7:0] Dataout | (Behavioral coding) The memory variable would be declared as: reg [7:0] datamemlarge [0:4095] |
| Stack256 | Clk Reset StackRead StackWrite [7:0] Datain | [7:0] Dataout | Partial structural coding, with the following important instances: 1 instance of SRAM256 2 instances of MUX4to1_8bit And some more instances of relevant modules to create the stack pointer/counter logic |
| ALU8 | Clk Reset [7:0] Operand1 [7:0] Operand2 [7:0] Opcode ALUSave ZflagSave CflagSave | Zflag Cflag [7:0] ALUout [7:0] ALUout2 | Any style ALUout2 used for storing the high-order byte during UMULT instruction |
| TimingGen | Clk Reset | T0 T1 T2 T3 T4 | Any style |
| ProgCounter | Clk Reset PCenable PCupdate [11:0] Address | [11:0] PC | Any style |
| INport | Clk Reset INportRead [7:0] InpExtWorld1 [7:0] InpExtWorld2 [7:0] InpExtWorld3 [7:0] InpExtWorld4 [3:0] Address | [7:0] Dataout | Any style |
| OUTport | Clk Reset [3:0] Address [7:0] Datain OUTportWrite | [7:0] OutExtWorld1 [7:0] OutExtWorld2 [7:0] OutExtWorld3 [7:0] OutExtWorld4 | Any style |
| ControlLogic | Clk Reset T1 T2 T3 T4 Zflag Cflag [7:0] Opcode | PCupdate SRAMRead SRAMWrite StackRead StackWrite ALUSave ZflagSave CflagSave INportRead OUTportWrite RegFileRead RegFileWrite | Any style |

- Each group will prepare one powerpoint based presentation (worth 10 minutes), and present their work using the projector in Lab, as per the slots (to be announced) on the dates (already in the list of lab sessions) in April 2026.

  - Each person of the group will have to speak.

  - It will be the responsibility of the group to check beforehand (1-2 days in advance) the appropriate connection (display settings and any adapters needed) of their laptop for the presentation.

- Each group will submit the following files by Monday 6-April-2026:

  - subcomponents.v

  - processor.v

  - The text file containing their assembly language code (in binary format)

    - (corresponding) hand-written assembly language code that was tested

  - Powerpoint presentation

The end